# Feature extraction

The Data used for this research is provided by Forschungszentrum Telekommunikation Wien (ftw). The database GeveAT is taken from the SpeechDat-AT, a telephone speech database for Austrian German [Baum2000]. The German speakers are taken from the SpeechDat-AT database as well, since the SpeechDat criteria allow 5% non-native speakers. The speech file format is 8bit, 8kHz, A-law speech files, uncompressed. 17 speakers from each Austria and Germany were provided. For each of the speakers 10 sentences were recorded. Every sentence was spoken by at least both an Austrian and a German speaker.

Because some of the speakers were not optimal for the task of discerning the origin of the speaker the data set was reduced to 10 speakers (7 male / 3 female).

The first task was to extract prosodically relevant parameters. As seen above, fundamental frequency and intensity contour are the most obvious features. In order to extract the intensity contour the a-law files were converted to wav-files. Duration features are much harder to obtain, because in this case needs segmentation information such as phoneme or syllable length are needed. Since these features had not been provided, duration features were not tested.

## Fundamental Frequency Tracking

Extracting the fundamental frequency of a speech signal can be achieved in several ways. For this task, Praat phonetics tools were used, which is a toolbox for speech research, including features as spectrogram, LPC, Cepstral-analysis, PSOLA, formant and pitch tracking. The pitch-tracking algorithm performs acoustic periodicity detection based on an accurate auto-correlation method, as described in [Boersma1993].

This method is more accurate, noise-resistant and robust than methods based on cepstrum or combs, or the original auto-correlation methods. Its key point is the fact that if one wants to estimate a signal’s short-term auto-correlation function on the basis of a windowed signal, the auto-correlation function of the windowed signal has to be divided by the auto-correlation function of the window.

### Theoretical Background

Ideally, the best candidate for the acoustic pitch period can be found using the position of the maximum of the auto-correlation function of the sound, the degree of periodicity from the relative height of this maximum. However, the problem is, that sampling and windowing cause inaccuracies concerning the position and height of the maximum.

The auto-correlation of a time signal x(t) as a function of the lag  is defined as:

rxτ≡x(t)x(t+τ)dt

If there is a maximum outside 0 and the height of the harmonic strength rx(max) is large enough the signal is periodic and in consequence there exists a lag T0, called the period. The fundamental frequency is then defined as F0=1/T0.

The short-term auto-correlation is estimated from a short windowed segment of the signal. This gives estimates F0(t) for the local fundamental frequency and R0(t) for the harmonic strength.

If there are strong harmonic components in the signal, the highest maximum of the auto-correlation of the windowed signal is rather at a lag that corresponds to the first formant than to the fundamental frequency. Therefore, the pitch estimate from the auto-correlation would be too high. A solution to this problem is to compute the normalized auto-correlation of the window function and to divide the auto-correlation of the signal by the auto-correlation of the window (See Figure 5.1).

x(t)	    multiplied by		w(t)		gives		a(t)

ra()	    divided by		rw()		gives		rx()

Figure 5.1: Pitch tracking using corrected autocorrelation (from [Boersma1993])

The corrected auto-correlation is:

rx(τ)≈ra(τ)rw(τ)

For every frame, a certain number of candidates (peaks in the auto-correlation function) are stored. A post-processing algorithm, considering cost for voicing threshold, octave jumps, voiced/unvoiced changes, etc., seeks the best path through the candidates.

The following arguments had to be applied:

Time step: the measurement interval, in seconds.

Minimum pitch: candidates below this frequency will not be recruited. This parameter determines the length of the analysis window.

The default arguments for the best path algorithm were used as suggested by praat. For male speakers the minimum frequency was 75 Hz and the maximum was 220 Hz. For female speakers the expected fundamental frequency was between 100 Hz and 350 Hz. For all samples a time step of 10 ms was used.

After the pitch tracking a smoothing algorithm was applied with either 8 Hz or 1 Hz bandwidth.

### Pitch-post-processing

Human pitch perception is rather logarithmical. Consequently, the logarithm of the fundamental frequency is taken and transformed to MIDI-Numbers (A2=110Hz=MIDI#36). This explains why peaks at the end of the downward trend (declination) of an utterance (see Error: Reference source not found) are perceived as strong as the higher peaks at the beginning.

For statistical analysis, the usual downward trend of an utterance is removed by subtraction of the 1st order regression line. This is also to reduce speaker dependencies, specially concerning gender.

## Parametric Description of the F0-contour

The following pages describe attempts to model the F0 contour with a parametric description. All of the models are originated in speech synthesis and they are used for modeling the pitch contour for synthesized speech.

### TILT-Analysis

The Tilt-Analysis is a phonetic model of intonation parametric representation of a pitch contour using three parameters for intonational events: Duration, Amplitude and Tilt (for the shape) [Taylor2000]. Intonational events can be either pitch accents (denoted a) or boundary tones (b).

The Tilt-Analysis is based on the rise/fall/connection (RFC) model. In this model, presented by [Taylor1995], each intonational event is characterized by four parameters: rise amplitude, rise duration, fall amplitude and fall duration. If an event has only a rise component, its fall amplitude and duration are set to 0. Likewise, when an accent only has a fall. The sections of contour between events are called connections (denoted c) and are also described by amplitude and duration.

In [Taylor2000] it is shown, that the RFC mechanism is not ideal in that the RFC parameters for each contour are not as easy to interpret and manipulate as one might like. Additionally, the parameters are highly correlated and therefore it is possible to reduce the set of parameters to three by transforming the four RFC parameters into three Tilt parameters, namely duration, amplitude and tilt itself.

A single parameter can be used to model the shape of the event. This tilt value is calculated:

tilt=Arise−Afall2Arise+Afall+Drise−Dfall2Drise+Dfall

The amplitude parameter is the size of the F0 excursion of the event:

Aevent=Arise+Afall

The duration is the sum of the rise and fall duration:

Devent=Drise+Dfall

F0 position is the F0 distance from the baseline (usually 0 Hz) to the middle of the event.

Time position is where the event is located in time.

+1.0

+0.5

0.0

-0.5

-1.0

Figure 5.2: Examples of 5 events with varying values of tilt

Using Tilt, the problem arising is that a label file for the intonational events is needed. This is normally created by hand, which cannot be consistently done by different labelers. I have not succeeded in finding an automatic labeling tool available.

### Intofit

Intofit is, like Tilt, a parametric description of F0-contours, originally to be used for speech synthesis. It is a maximum-based model, assuming the F0-maxima to be the most important points of the intonation contour [Heuft+1995]. Each F0-contour is parameterized describing only its maxima: for each maximum, approximated by cos2 functions, four parameters are given. First, the maximum is located precisely in time, relative to the onset of the accented vowel assigned to it. This distance is called delay.

The second parameter is, the height of the maximum (amplitude) is described as a percentage value between top and baseline. The third and forth parameter describe the steepness of the contours preceding (left slope) and following (right slope) the maximum.

AMPLITUDE

=1

left

slope

right

slope

left

slope

right

slope

delay

(positive)

delay

(negative)

AMPLITUDE

=0,75

topline (speaker dependent; fixed)

baseline (speaker dependent; fixed)

begin stressed vowel

begin stressed vowel

Figure 5.3: Intofit parameters (from [Heuft+1995])

The intonational events for Intofit were obtained assuming all accents were represented by pitch maxima down to a certain threshold. This is linguistically not correct, that is why the delay parameter is always set to zero, but the F0-contour could be approximated quite correctly (Figure 5.4).

In Table 5-1 the Algorithm for finding the Intofit parameters is described. For the slope parameters an optimization by minimizing mean squared errors was performed. A weighing factor emphasizes the distance close to the maximum, because deviations close to the minimum are perceptional less relevant.

  - Finding the maximum close to the accented vocal

  - Calculation of distance between vocal onset and position of maximum

  - Determining of relative amplitude, related to top and baseline

  - Calculation of optimal slope parameter between previous minimum and current maximum.

  - Calculation of optimal slope parameter between current maximum and the following minimum.

Table 5-1: Calculation of Intofit parameter (from [Portele+1995])

It is to mention though, that quite often the approximation algorithm didn’t find a correct fitting. Most of the time, the peaks of the fitted-contour were below the original peaks.

Figure 5.4: Original and (Into-)fitted F0-contour (log-domain)

Originally, Intofit uses linear frequency, but because speech is perceived rather logarithmically, a logarithmical input was used as well. Then for every speaker one mean intofit feature set was calculated (see Figure 5.5).

Figure 5.5: Log- Intofit-features: amplitude vs. falling slope

### The Fujisaki - Model

The Fujisaki-model ([Fujisaki1983] and [Mixdorff1997]) aims at modeling the generation process of F0 and explaining the physical and physiological properties behind it. It views a F0 contour as the filtered sum of two components: word level accent commands and phrase-level utterance commands. Thus the F0 contour, F0(t), of a sentence can be expressed by

lnF0(t)=lnFmin+i=1IApiGpi(t−T0i)−Gpi(t−T3i) +j=1JAajGaj(t−T1i)−Gaj(t−T2i)	Equation 5-1

where

Gaj(t)=[1−(1+βjt)e−βjt]u(t)	Equation 5-2

and

Gpi(t)=αite−αtu(t)	Equation 5-3

u(t) = unit step function

respectively indicate the step response function of the corresponding control mechanism to the phrase and accent commands. The i’s and j’s are expected to be fairly constant within a sentence, or among utterances of an individual speaker. I and J are the number of phrase and accent commands, T0i and T3i denote the onset and end, respectively, of the ith phrase command, while T1j and T2j denote the onset and end, respectively of the jth accent command. In the absence of pauses within a spoken sentence, the offset times T3i for all phrase commands are assumed to be the same for all i’s within an utterance. On the other hand, the accent commands are constrained not to overlap each other.

t

Figure 5.6: Block diagram of the Fujisaki-model

To extract the features the program ‘autofuji’ by Hansjörg Mixdorff was used [Mixdorff1999]. Fujisaki-model parameters are estimated from an ESPS-waves-based f0 contour in a multi-step procedure, consisting of a quadratic spline stylization, a component separation by filtering followed by command initialization. Then the initial parameter configuration is optimized in a three-pass hill-climb. In the latter part of the procedure, parameters for accent and phrase components are optimized separately first, then further optimized together using the spline contour as the target and ultimately fine-tuned with a weighted version of the extracted contour as the target (See Figure 5.7).

high frequency component

accent component

low  frequency

component

phrase component

synthetic signal

original

F0-contour

Figure 5.7: Components of Fujisaki Model

Several possible ranges for  and  were evaluated and finally =2 and =20 yielded the best results. [Mixdorff1983] already suggested those values for German.

This leaves two variables Aa and Ap as features for the identification of the different dialect groups. Since there can be several accent and phrase components per utterance, representative values for each sentence were calculated. Best results offered a median computation so that there is one median Aa and Ap value for every sentence (Figure 5.8).

For some sentences, the Ap output of the autofuji-program was not calculated, which is obviously wrong. Additionally some abnormal program terminations have occurred. Those errors may be caused by the missing voicing degree, which was not available. However, the involved datasets were still used, because Aa was accurately calculated. This was taken care of by setting the median value of the invalid Ap components to zero.

Figure 5.8: Fujisaki Parameters Ap vs. Aa

### LPC-Coefficients

This approach is also rooted in intonation modeling for speech synthesis. [Mersdorf1999] proposed a system where LPC-Coefficients for speaker dependent modeling of intonation were used. The LPC-Intonation model consists of the following stages:

- F0-postprocessing: Outliners are removed

- Interpolation: For the LPC analysis a continuous, derivable representation of the F0 contour is recommended. They suggest a cubical spline interpolation assuming a ‘virtual F0’ in unvoiced segments (see Figure 5.9). The interpolation is motivated by the assumption that temporary switching into unvoiced excitation only interrupts a continuous speaker’s intonational gesture [Mersdorf1997].

F0

virtual F0

j   e   n   @     s

j   e   d     @     s

t

Figure 5.9: Example for vitual F0 (jenes, jedes); from [Mersdorf1997]

- Analysis: The analysis consists of an 8th order LPC of the interpolated contour over the whole sentence. For the whole speech material a single set of individual filter coefficients can be built by computing the arithmetical mean value for each coefficient.

- Approximation of command excitation and (re)synthesis are then used to generate an excitation signal for the resynthesis of a synthetic intonation contour. This can then be applied to synthetic speech using PSOLA or similar techniques.

In Figure 5.10 it is overt that there are quite significant differences in the impulse response of different speakers using 8th order LPC coefficients.

Figure 5.10: Speaker individual Impulse responses (Austrian speakers)

These are mean impulse responses for individual speakers. The idea proposed now is that significant differences between Austrian and German speakers can be expected. Instead of averaging over single speakers mean filter parameters are calculated for all Austrian and German speakers.

Figure 5.11: Average Austrian and German impulse response

Figure 5.11 shows that the differences are far less significant than with single speakers.

The frequency response (Figure 5.12) does not show any differences either.

Figure 5.12: Frequency response of average LPC-Coefficients

Further analysis proved that LPC-Coefficients provide no useful features for the discerning of Austrian and German.

### Peaks and Intervals

The most simple form of obtaining a data reduced representation of the pitch contour is to find minima and maxima. This was done in the log-domain; therefore intervals are given in semitones.

Figure 5.13: minima & maxima of F0-contour

The histogram of the mean intervals between a maximum and a minimum suggests a possible distinctive feature (Figure 5.14.).

Figure 5.14: Histogram of mean Intervals

## Intensity

Additionally, an intensity contour was calculated in MatLab with the following algorithm (RMS):

Xn=1Ni=n−79n+80xi2	Equation 5-4

The values of the sound wave are squared and then summed over a window length of 160 samples (20ms). The hopsize used was 10 ms (80samples).

From the linear intensity contour the logarithm was taken {20*log10 (X[n])}. For normalization the maximum of each file was set to 80dB. It was then stored as a Praat Intensity-Tier file.

## Statistical Features

Very little useful information is available from intonation research, which could lead to more knowledge-based features. Statistical features might be a possible approach.

Percentiles (P10, 25, 50, 75, 90), standard deviation, skewness, kurtosis were calculated for the following signals:

F0-Contour: The logarithm was taken, converted to MIDI-numbers and to detrend the data (see Section Error: Reference source not found) the first regression line was subtracted.

Delta F0-Contour: It is said that a lot of prosodic information lies in slopes and intervals (see Section Error: Reference source not found), so delta-F0 could provide some useful information.

- Intensity-Contour (framesizes=160/20ms): Intensity is said not to be very useful according to [Thymé-Gobbel+1996], but there is still some information encoded in the intensity contour. Because of background noise the file is much longer than the actual speech. This fact was taken into account by assuming that the actual speech-length is from the first to the last voiced frame. This segment was used to analyze the intensity-contour.

- Delta Intensity-Contour (framesizes=160/20ms): The length was calculated as above and the difference was computed.

- AutoCorrelation(F0): Auto-correlation provides spectral information about the F0 contour. This function was computed on the logarithmical and detrended F0-contour.

- AutoCorrelation(Intensity): The auto-correlation function was computed on the logarithmical and detrended Intensity-contour

- CrossCorrelation(F0,Intensity (lin/log)): Cross-correlation shows dependencies between F0 and intensity contour. It is expected that Austrian and German show different patterns in interaction between F0 and intensity.

- F0*Intensity-Contour (lin/log): This was computed as just an additional feature that might show dependencies between F0 and intensity.

Voiced Ratio: One widely acknowledged difference between German and Austrian pronunciation is the voicing of consonants [see Section Error: Reference source not found]. Austrians rarely use voiced consonants such as [, , ,…], but substitute them with their voiceless counterpart. Even though this is not a clearly prosodic feature, it is assumed that in German speech the voiced rate must be higher than in Austrian utterances. Two different approaches were made. First the Praat voicing decision was utilized by computing the ratio of the number of voiced frames to the number of all frames of an utterance. An alternative way was the computation of the zero-crossing rate (see Section Error: Reference source not found)

## Summary

This chapter explored how to get features relevant for classification of Austrian and German using prosody. Two acoustical characteristics are explored, first fundamental frequency and second intensity. Two different parameterizations of fundamental frequency are applied, Infofit and Fujisaki. Both offer interesting results.

For both F0 and intensity several signals such as delta, correlation, etc are calculated and then statistically evaluated, using the t-test.

Processing time of the most important calculations is shown in Table 5-2. The used system was an Intel Pentium III – 500 MHz with 192 MB RAM. Values are in % of real-time. It has to be noted, that the statistical features from Section 1.4. are calculated in MatLab, which is rather slow. Optimizations could lower the processing time.

| Task | processing time % of real-time |
| --- | --- |
| Pitch Tracking | 33% |
| Intofit | 10% |
| Fujisaki | 40% |
| Statistical Features (for each signal) | <5% |

Table 5-2:Processing time

See a summary of the feature extraction in Figure 5.15.

Figure 5.15: Summary of feature extraction

Vienna Telecommunication Research Center

Praat can be obtained from: http://www.fon.hum.uva.nl/praat/

Tilt is part of the Edinburgh Speech Tools Library, which is provided by the Centre for Speech Technology, University of Edinburgh. It can be obtained from http://www.cstr.ed.ac.uk/projects/speech_tools.html.

Intofit was developed at the ‘Institut für Kommunikationsforschung und Phonetik’ at the University of Bonn, Germany and can be obtained from http://www.ikp.uni-bonn.de/~tpo/intofit.html .

Since ESPS-waves was not available, the missing degree of voicing parameter was substituted with a binary value. This might decrease the performance achieved.