# Speech processing

Speech processing has become a broad field of research including many specific areas, which are sometimes closely related. This chapter should introduce the main areas of what speech processing is all about. Most of the material is taken from [O’Shaughnessy2000] if not noted differently. The book is an excellent introduction to speech processing provides extensive references and thus can be used for more in-depth studies.

The following topics are covered:

- Speech Analysis

- Speech Synthesis

- Automatic Speech Recognition

- Speaker Recognition

- Language Recognition

- Accent/Dialect Recognition

## Speech Analysis

Speech Analysis is maybe the technical foundation for all other disciplines. It involves a transformation of a speech signal s(n) into another signal, a set of signals or a set of parameters, with the objective of simplification and data reduction.

Speech analysis tries to extract relevant features while suppressing redundancy or irrelevance and finding efficient representation of speech. Since speech analysis cannot be covered on a few pages, only methods that were actually used during this research are mentioned.

There are a few main assumptions about speech signals.  First, it is usually assumed that the signal properties change relatively slow with time.  This allows examination of speech with short-time windows presuming the parameters remain constant for the duration of the window. Usually speech sound is assumed to stay constant for at least 10ms. This opens the whole windowing subject, which is not to be explored here. Background can be found in [Oppenheim+1995].

In the time-domain, ana  lysis transforms a speech signal into a set of parameter signals, which usually vary much more slowly in time than the original signal.

The Zero-crossing rate (ZCR) provides very simple analysis in the time domain for spectral measures. In a signal s(n) such as speech, a zero-crossing occurs when s(n)=0, i.e., the waveform crosses the time axis.  For narrowband signals (e.g. sinusoids), ZCR is an accurate spectral measure.

The ZCR can be defined with

Tsn=0.5sgn(s(n))−sgn(s(n−1)),

where the algebraic sign of s(n) is

sgn(s(n))=1 for s(n) ≥0−1 otherwise

The ZCR can help in voicing decisions. Most energy in voiced speech is at low frequency, since the spectrum of voiced glottal excitation decays at about –12 dB/Oct. In unvoiced sounds, broadband noise excitation excites mostly higher frequencies, due to effectively shorter vocal tracts. While speech is not a narrow-band signal (and thus the sinusoid example does not hold), the ZCR correlates well with the average frequency of major energy concentration. Thus, high and low ZCR correspond to unvoiced and voiced speech, respectively.

Short-time energy or amplitude can help segment speech into smaller phonetic units, e.g. approximately corresponding to syllables.

E[k]=m=0N−1xm2wn−m

Short-time autocorrelation gives information about energy and periodicity of the signal. It is used for F0 determination and Linear Prediction.

Rnk=m=−∞∞smwn−msm−kwn−m+k

Frequency domain parameters provide the most useful parameters for speech processing. The basic model of speech production is a noisy or periodic waveform that excites a vocal tract filter. This corresponds well to separate spectral models for the excitation and for the vocal tract (see Figure 3.1).

Figure 3.1: source-filter speech model

Human hearing appears to pay much more attention to spectral aspects of speech than to phase or timing aspects. Thus, spectral speech analysis is paid much more attention to.

Short-Time (discrete) Fourier transform (STFT) applies the discrete Fourier transform (DFT) to successive windows:

Sk=m=0N−1sme−j2πkmNwn−m

The choice of N (window length) is crucial for STFT. Low values for N give poor frequency resolution, but good time resolution. Large N, on the other hand, gives poor time resolution and good frequency resolution. As an optical speech analysis tool the spectrogram provides a three dimensional representation of speech utterances using the STFT. For speech analysis there are two main representations. Wideband analysis displays individual pitch periods as vertical striations corresponding to the large amplitude at vocal cords closure. It smoothes the harmonic amplitudes under each formant across a range of 300Hz, displaying a band of darkness for each formant. The center of each band is a good estimate of formant frequency.

Narrowband spectrograms display separate harmonics instead of pitch periods.

They aid analysis of F0 and vocal tract excitation (see Figure 3.2).

Figure 3.2: Wideband and narrowband spectrograms of a sentence

Linear Predictive Coding (LPC) is one of the most powerful speech analysis techniques, and one of the most useful methods for encoding good quality speech at a low bit rate. It provides accurate estimates of speech parameters, and is relatively efficient for computation. The underlying assumption of LPC is a speech model as in Figure 3.1, which is a source-filter model with an excitation signal of either an impulse train or random noise. This excitation signal is then filtered by the vocal tract transfer function.

H(z)=S(z)E(z)=G1−k=1Pakz−k

The predicted signal is calculated using an FIR filter:

xn=k=1Pαkxn−k

Figure 3.3: Block diagram for linear prediction

Linear predictive analysis is a technique aimed at finding the set of prediction coefficients {k} that minimize the mean-squared prediction error between a signal x[n] and a predicted signal based on a linear combination of past samples; that is

fn2=xn−k=1Pαkxn−k2,

where  represents averaging over a finite range of values of n. Most of the time, the autocorrelation method is used to find the optimum predictor coefficients {k}. Small segments of speech (usually approx. 10ms) are used to ensure that the signal doesn’t change significantly during analysis.

If the speech signal x[n] is filtered by an inverse or predictor filter (the inverse of an all-pole V(z))

A(z)=1−k=1Pαkz−k

the output e(n) is an error signal

fn=xn−xn=xn−k=1Pαkxn−k≡Gen.

This signal (called residual) as input will synthesize the signal perfectly.

xn=k=1Pakxn−k+Gen

From f[n] it can be determined whether the signal is voice or unvoiced and if voiced the fundamental frequency. This is how basic LPC encodes the residual; tough that causes quality loss, because the predicted filter is never ideal.

Problems arise when the excitation is not purely voice or unvoiced, but something in between. Various efforts have been made to code the error signal. For example, CELP (Code Excited Linear Prediction) is a version where the residue signal matches entries in a codebook and uses the code for that entry.

Determining the fundamental frequency (F0) is important in many speech applications. It is the primary acoustic cue to intonation and stress in speech. Most low-rate voice coders require accurate F0 estimation for good reconstructed speech. F0 patterns are useful in speaker recognition and synthesis. Time domain F0 detectors have three components: a preprocessor  (to filter and simplify the signal via data reduction), a basic F0 extractor (to locate pitch epochs in the waveform), and a postprocessor (to locate errors).

Frequency-domain methods for F0 estimation involve correlation, maximum likelihood, and other spectral techniques where speech is examined over a short-term window. In Section Error: Reference source not found an auto-correlation F0 estimation method will be discussed.

## Speech synthesis

Text-to-speech synthesis (TTS) is the automatic generation of a speech signal, starting from a textual (or conceptual) input and using previously analyzed digital speech data.

Two main steps are required, first the linguistic analysis transferring a text to a rather phone orientated description including prosodic information and second the actual speech wave generation.

The text analysis consists of different stages (see Figure 3.4). The preprocessor does the normalization and segmentation of the text. Numbers, special symbols, abbreviations, acronyms, control characters, etc. have to be handled. Punctuation is examined as a cue for sentence end detection.

Figure 3.4: Text analysis for speech synthesis (from [Dutoit1997])

The morphological analyzer decomposes all words into their elementary units (Morphemes) to be able to have a dictionary at a reasonable size.

The syntactic analysis helps to identify the part-of-speech of every word and can then structure the text or sentence to be able to extract prosodic information.

Semantic information would help to solve most of the current problems, but so far and probably not in the near future there is no way to find this out.

The Letter-to-sound module then uses the already obtained information for the phonetization of the text.

The most difficult part is to generate proper prosody for unrestricted text. This is one of the main reasons that synthetic speech still doesn’t sound natural.

Speech-wave production methods can be broadly divided into two categories. – those which predominantly model the speech signal and those which predominantly concatenate prerecorded speech signal.

There are two prominent members of modeling speech – ‘articulatory synthesizers’ and ‘formant synthesizers’. Both generate a synthetic speech signal purely from parametric information driving an abstract model of production.

Articulatory synthesizers attempt to produce a synthetic speech by modeling the characteristics of the vocal tract and the speech articulators. There is still not enough information on how the speech process is exactly going on, partly because there are no appropriate models of the speech production available. Research is going on, but no commercial use is made so far, because great computational power is needed for real time applications.

Formant synthesis looks at the acoustical properties of speech and tries to model them. The underlying model is most of the times a source-filter-approach (Figure 3.1).

The second approach is to encode pieces of natural speech and put them together as needed. The advantage is that no complicated model of the speech signal is necessary and with the possibility of advanced techniques for modifying the signal and cheap computer memory concatenative synthesis is now the common approach for commercial TTS Systems.

## Automatic Speech Recognition

Automatic speech recognition (ASR) has been much more difficult to achieve than TTS. When trying to let a computer recognize human speech several problems arise, such as variability (time, speaker, etc.), vocabulary, continuous speech, etc.

In theory, ASR could be as simple as a large dictionary where each entry is a digitized stored waveform labeled with a text pronunciation. Given an input utterance, the system would just search the dictionary for the closest match and find the corresponding text from a lookup table. Whereas this approach works for speaker dependent discrete utterance small vocabulary application, for more complex systems this procedure is not feasible due to immense complexity. Different applications yield very different complexity of the ASR system.

|  | Discrete utterance | Connected speech | Continuos speech |
| --- | --- | --- | --- |
| Speaker Degree of complexity Dependent |  |  |  |
| Multi Speaker |  |  |  |
| Speaker Independent |  |  |  |

Table 3-1: Complexity of different recognition tasks (From [Morgan+1991])

The many sources of variability in human speech are a major reason for this complexity. All recognizer are influenced by environmental variability due to background and channel noise (specially for phone applications) Speaker-dependent recognizers have the least variability, but still every word is not pronounced the same all the time (Intra-speaker variability). It still takes training time to adapt the system to a specific speaker. On the other hand, speaker-independent systems have to cope with inter-speaker variability. Here many factors are implicated such as gender, age, accent or dialect, style of speaking and different anatomy of the vocal tract. Those sources of variability are the key problems of ASR.

Excursion on Problems of Dialect and Accent in ASR

With the example of German, the different influences of dialectal coloring on ASR will be considered. Dialectal differences are a major problem in ASR. Depending on the region realization of phonemes and so pronunciation of words differ a lot. Not only acoustic deviations, but also lexical differences make it necessary to include many dialectal regions into an ASR system. [König1981] described several dialectal subdivisions and boundaries.

Foreign accents may have the problem that phonemes of the target language do not exist in the original language, so people might not be able to pronounce those correctly

To keep down complexity, which is rising with utterance length, an important task is to segment speech into smaller units such as words syllables or phones. However, it is hard to find reliable cues for this task. Sudden large changes in speech spectrum or amplitude help to estimate unit boundaries. For example, silence can be between words, but it can also be before plosives or at glottal stops. Correct endpoint detection helps to improve error rates and keep down computational costs.

For performance evaluation error rates (e.g. the percentage of words not correctly recognized of those spoken) or accuracy are used. Cost, speed and the likelihood of an input being rejected are other important factors.

Speaker dependent isolated word recognition with a small vocabulary often reach accuracy of >99%, but fall to 90-95% for speaker independent connected speech applications.

One crucial tool for speech recognition as training data and evaluation tools are databases of speech labeled with textual transcriptions. For German the Bavarian Archive for Speech Signals (BAS) [Schiel+1999] provides different corpora including RGV1 (A Database for Regional Variants of German) [Burger+1998] and SpeechDat project which includes an Austrian corpus [Baum+2000].

The crucial point of speech recognition is the problem of pattern recognition. An utterance has to be compared with reference data obtained through training. This data can be divided into two main approaches. One can view ASR from either a cognitive, which is a knowledge based or expert system, or an information theoretic view. The first one tries to model through finding relationships between speech signals and their corresponding text messages and postulating phonetic rules to explain the phenomena. It is very difficult to model the complexity of speech in a knowledge-based system.

The information theoretic view tries to describe speech with information derived through statistic analysis. It uses statistical models that maximize the likelihood of choosing the correct symbols corresponding to the input.

### A General Model for Speech Recognition

A general speech recognition model is illustrated in Figure 3.5. The major components of this model include:

- Preprocessing is to normalize the speech signal

- Parameterization and feature extraction is for identifying the key components of a parametric representation and eliminating redundant information.

- Time alignment and pattern matching algorithm for performing word detection

- Language processing is for selecting a linguistically valid word string

Pre-processing

Feature

extraction

Time alignment

and

Pattern matching

reference

template

Legal text

sequences

Phonetic

dictionary

Word

hypotheses

Linguistic

constraints

Word

verifying

speech

Text output

A priori

training

Figure 3.5: General speech recognition model

#### Preprocessing

First, the speech utterances have to be normalized for example with automatic gain control (AGC) for reduction of the influence of different recording conditions (for example through different distance to the microphone) or transmission channels. It has to be applied with long time constants to preserve prosodic information in amplitude changes. Normalization of temporal variations is not done at this level.

#### Parametric representation and Feature extraction

The parameterization of the speech signal has the goal of efficient data reduction without losing information relevant for ASR.

#### Pattern matching and time alignment

At the heart of ASR lies the measurement of similarity between two windowed speech patterns, i.e. the representation of a frame of the input speech and one from a set of reference patterns or models (obtained during training). The comparison or evaluation involves finding the best match in terms of a distance between templates or deciding which reference model is the most likely.

#### Linguistic Evaluation

Most speech corresponds to texts, which follow linguistic rules (e.g. lexical, syntactic, semantic constrains). Exploiting these rules is crucial for ASR performance.

### Parametric representation and Feature extraction

Usually successive frames of 10ms distance (see Chapter 1.1. ) are parameterized.  Usually a source-filter model as in Figure 3.1 is used for the parameterization. But very often the excitation parameters as the voicing decision, amplitude and pitch are ignored in ASR, though recent research tries to employ those prosodic parameters to aid recognition at a linguistic level (Figure 3.6). [Nöth+1997] use a combination of prosodic information such as fundamental frequency and energy, and a word hypothesis generator following a phoneme recognizer to gain positions of accents and sentence boundaries. [Strom1995] uses only Energy and F0 of a speech signal to place the accents and sentence boundaries. Both are part of VERBMOBIL [Walster1996] a multidisciplinary research project in Germany. Its goal is to develop a tool for machine translation of spoken language for German to English and Japanese.

Figure 3.6: Incorporation of prosody for speech recognition (from [Nöth1997])

The spectral envelope provides the primary ASR parameters. The most common ASR parameters are mel-based cepstral coefficients, but LPC analysis, energies from a channel vocoder, reduced forms of DFT and zero-crossing rates in bandpass channels are other examples. They all attempt to capture in about 10 parameters enough spectral information to identify spoken phones.

Now these parameters can be reduced to features to decrease redundancies of the parameters. Such features may be subdivided into acoustic and phonetic features, depending of the degree of data reduction. Phonetic features have a discrete range and assign sound to linguistic categories, e.g. voiced or fricative; they represent major data reduction toward a phonemic decision. Acoustic features (e.g. formants, F0) represent an intermediate step between parameters and phonetic features.

Features are fewer in number than parameters and therefore potentially more efficient for ASR; they are speech specific and require classification that can be erroneous.

### Pattern matching and time alignment

Each parametric (or feature) pattern for a frame of speech can be viewed as an N-dimensional vector, having N parameters/frame. If the parameters are well chosen, then separate regions can be established in the N-space for each segment.

The similarity between two patterns is often expressed via a distance, measuring how close the patterns are in N-space. Another popular method is statistical, where the reference models store probability density functions (PDF) and similarity is judged in terms of the likelihood of the test pattern for each PDF. To handle multiframe utterances (i.e. all practical cases), local (frame) distance measures typically sum to yield a global (utterance) distance. The reference pattern yielding the smallest distance or highest probability is usually chosen for the ASR output.

##### Euclidean and Mahalanobis distances:

In ASR involving templates, each unknown test utterance is converted to an N-parameter test template, to be compared against reference templates to find the closest match. The similarity of two templates is inversely proportional to the distance in N-space between points corresponding to the templates. The most common distance measure is the Euclidean distance (or L2-norm):

d2(x,y)=(x−y)T(x−y)=i=1N(xi−yi)2

Another common speech distance in the Mahalanobis or covariance-weighted distance,

d(x,y)=(x−y)TW−1(x−y),

where W is a positive-definite matrix that allows different weighting for individual parameters depending on their utility in identifying the speech segments in N-space.

Despite the advantages of the Mahalanobis distance in weighing properly, ASR often uses the Euclidean distance or a LPC distance, because in is difficult to reliably estimate W from limited training data and the latter two distances require only N multiplications for an N-dimensional parameter vector vs. N2 multiplications with the Mahalanobis distance.

##### Stochastic similarity measures:

The Mahalanobis distance has origins in statistical decision theory. If each utterance of a word represents a point in N-space, the many possible pronunciations of that word describe a multivariate PDF in N-space. Assuming ASR among equally likely words and maximum likelihood as the decision criterion, Bayes’ rule specifies choosing the word whose PDF is most likely to match the test utterance. Because of the difficulty of estimating accurate PDFs from a small amount of training data, many systems assume a parametric form of PDF, e.g. Gaussian, which can be simply and fully described by a mean vector µ and a covariance matrix W. Since ASR parameters often have unimodal distributions resembling Gaussians, the assumption can be reasonable.

The distance measures in the previous paragraph are general and can be used with many sets of parameters such as LPC or Cepstral parameters.

##### Dynamic time warping

The dynamic time warping (DTW) procedure combines alignment and distance computation in one dynamic programming procedure. DTW finds an optimal path through a network of possibilities in comparing two multiframe templates, using the Bellman optimality principle. DWT aligns a test template as a whole with each reference template by finding a time warping that minimizes the total distance measure, which sums the individual frame distances in the template.

T(n)

T

n

T

n

1

1

1

m=w(n)

R

R

m

m

1

R(m)

Figure 3.7: Dynamic time warping from [O’Shaugnessy2000]

### Networks for speech recognition

Networks in ASR employ a rather statistical representation of acoustic information. Have isolated word recognition (IWR) as an example: model each word with a succession of phonetic states i (corresponding roughly to phones), linked by transitions specified by likelihoods aij. This probability of a phonetic segment j following segment i governs the transition between the states representing those two sounds. Consider pass as an example word, where states for [] closure (silence), [] burst, [] aspiration [], and [] might be chosen via coarse segmentation or other technique. To allow for the chance that the [] burst and/or aspiration may be missing the aij may vary considerably, the typically correspond to the frequency of actual transitions in the training data.

For IWR each input utterance is evaluated by each word network, to find the network most likely to have generated the word. Instead of searching a DTW space for a path of minimal distance, each network is searched for the path maximizing the product of all transition probabilities between states corresponding to the test utterance.

Popular approaches are Hidden Markov Models [Rabiner1989] or Neural Networks [Morgan1991].

Hidden Markov Models

Rabiner presents an excellent introduction to Hidden Markov Models (HMMs) in his tutorial [Rabiner1989].

The key assumption of the statistical approach to speech recognition is that speech can be modeled statistically during an automatic process. By examining an ensemble of training speech data, a probabilistic that characterizes the entire ensemble is created. The resulting model, which represents each speech unit (word or sub unit), is more powerful and general than a template.

In the HMM formalism, speech is assumed to be a two-stage probabilistic process. In the first part of the two-stage process, speech is modeled as a sequence of transitions though states. The states are not themselves directly observable (hidden), but are manifest by observations, or features. Second, the observations in any state are not deterministic, but are specified by a probabilistic density function over the space of features. The power and flexibility of the statistical approach derives from this two-stage modeling procedure.

Figure 3.8: 3-state Hidden Markov Model

As shown in Figure 3.8, a HMM is characterized by the following:

- N, the number of states in the model. We denote the individual states as S={S1, S2,…, SN}, and the state time t as qt.

- M, the number of distinct observations per state. We denote the individual symbols as V = {v1, v2, …, vM}.

- The state transition probability distribution A0{ai,j}where

ai,j =  P [ q t+1 = Si | q t = Sj ]	1  i, j  N

For the special case where any state can reach any other state in a single step, we have ai,j > 0 for all i,j. This is called an ergodic HMM. In ASR left-right HMMs are very common because time depended properties can be modeled very well. 
Only state transitions from left to right are allowed (Figure 3.9).

Figure 3.9: Left-fright HMM

- The observation symbol probability distribution in state j, B={bj(k)}, where

Bj(k) = P [vk at t | qt = Sj ]	1 j  N

1 k  M

- The initial state distribution  = {i}, where

i = P [ q1 = Si ]		1 j  N

Given appropriate values of N, M, A, B and , the HMM can be used as a generator to give an observation sequence:

O= O1 O2 … OT

For convenience normally a compact notation for a HMM is used:

 = (A, B, )

Given a HMM there are three basic problems of that must be solved for the model to be useful in real-world applications:

Problem 1: 	Given the observation sequence O= O1 O2 … OT, and a model  = (A, B, ), how do we efficiently compute P(O|), the probability of the observation sequence, given the model.  The Baum-Welch forward-backward algorithm may be used to find the probability P(O|Mk) of generating the observation sequence O from the model Mk

Problem 2: 	Given the observation sequence O= O1 O2 … OT, and a model , how do we choose a corresponding state sequence Q=q1, q2, …, qT which is optimal in some meaningful sense  (i.e., best ‘explains’ the observations)?  There is a well know Viterbi dynamic programming solution.

Problem 3: 	How do we adjust the model parameters  =(A, B, ) to maximize P(O|)  The model parameters are found by an iterative procedure known as Baum–Welch re-estimation. An initial HMM is assumed, and the Baum-Welch forward-backward algorithm is carried out to find the state occupation probabilities as a function of time.

HMMs can be used to model phones, words or larger speech units. A phone HMM might use three states to represent to, in order, an initial transition spectrum from a prior phone, a spectrum from the phone’s presumed steady state, and a final transition spectrum to a following phone.

### Language modeling

Language models (LM) appear on a lexical (or phonotactic), a syntactic and possibly on  a semantic level.

Since the phonemic composition of words in most languages is highly restricted (e.g., the sequences /tz/ and /sd/ are illegal in English syllables), a word level language model is applied. Normally given a history of prior (recognized) words in an utterance, the number of words P that an ASR must consider as possibly coming next is much smaller that the vocabulary size V. P is called the perplexity of a language model. LMs are stochastic descriptions of text, usually involving likelihoods of local sequences of N consecutive words in training texts.

n-gram Models

Typically, N-gram models estimate the likelihood of each word, given the context of the preceding N-1 words, e.g. bigram models use statistics of word pairs and trigrams model word triplets. Unigrams are simply prior likelihoods for each word, independent of context. These probabilities are determined by analysis of much text, and are incorporated into a Markov language model.

### Summary

We have covered some basic principles of speech recognition. Even though only some topics were covered, the complexity of the task is obvious.

Most current ASR uses statistical pattern recognition, applying general models as structures to incorporate knowledge about speech in terms of reference models. The parameters of the models are estimated during a training procedure, in which speakers utter words or sentences, which may be repeated during actual ASR.

Alternative approaches such as cognitive methods, which incorporate knowledge on human speech production and perception, decreasing hardware cost will improve future ASR performance.

## Speaker Recognition

There are two main Speaker recognition applications:

- Verifying a persons identity prior to admission to a secure facility or to a transaction over the telephone

- Association a person with a voice, e.g. in audio-conferences

Other applications could include identifying the persons gender, emotions in speech [Waibel1996], accent or dialect of a speaker and the language being spoken. The latter will be covered extensively in Chapter Error: Reference source not found

There are two main areas in speaker recognition, first automatic speaker verification (ASV) and second automatic speaker identification (ASI). ASV only has to evaluate the test pattern with one reference model and a binary decision whether the test speech matches the model of the claimant. ASI, though, requires choosing which of N known voices best matches a test voice.

When focusing on the identity of a speaker there are three source of variation among speakers: differences in vocal cords and vocal tract shape, differences in speaking style (including variations in both target positions for phonemes and dynamic aspects of coarticulation such as speaking rate), and differences in what speakers choose to say.

Two classes of error occur: false acceptance when the system incorrectly accepts an impostor during ASV or identifies a wrong person during ASI, and false rejections, when the system rejects a true claimant in ASV or incorrectly finds no match in ASI.

Analysis techniques are similar for speech and speaker recognition since both involve pattern recognition of speech signals, but in speaker recognition only one decision is necessary compared to ASR where decisions are made for every phone or word. Templates or models are not focused on text, but on speakers. Features include prosodic properties, LPC and cepstral coefficients.

Main approaches are either utilizing feature using long time averages (e.g. means and variances of F0, amplitude or LPC coefficients) or comparing specific sound with a test template of e.g. phones. Two other categories are text dependent or text independent solutions.

### Prosodic cues for speaker identification

[Carey+1996] have utilized prosodic features based on pitch and energy contour for speaker identification. Gender was identified with 98% accuracy using the mean pitch parameter alone. Consequently, mean pitch was also used for identification of unknown speakers.

Additionally they used the first four statistics, mean, variance, skewness, kurtosis, of the pitch and energy and their first two derivatives. The mean and variance of the length of the voiced speech segments were added to these. Those features were tested to draw the seven best ones for classification using Linear Discriminant Analysis.

They then combined the prosodic system with a system using spectral envelope parameters using a filterbank containing 19 filters. The log power outputs were transformed into twelve cepstral coefficients and twelve delta cepstral coefficients at a frame rate of 10 ms. Hidden Markov Models were used for classification. The spectral envelope yielded better results than the prosodic features. However, the latter were much more robust to signal degradation than the spectral envelope.

[Waibel1996] used two sets of features to recognize the emotional state of a speaker. The first set consists of 7 global statistics of the pitch signal such as mean, standard deviation, minimum, maximum, range, slope and speaking rate.

The pitch contour for the second feature set was smoothed using piecewise cubic splines. The derived features were statistics related to rhythm, the smoothed pitch signal and its derivative, individual voiced parts and slopes.

ASV/I will be used for discussions applying both to ASV and ASI